Bicycle Plan

City of Greenwood, Arkansas

Purpose: To develop a safe, efficient, and economical comprehensive bicycle facilities plan for the city of Greenwood, which will become a specific element of the city’s

Master Street Plan and will be implemented through the appropriate city codes and ordinances.  This plan includes routine accommodation of bicycling as a fundamental travel mode and is an integral component in the planning, funding, design, construction, operation, and maintenance of Greenwood’s transportation network.  "Accommodation" is broadly defined as "any facility, design feature, operational change, or maintenance activity that improves the environment in which bicyclists travel."

Authority: The City’s authority to develop, adopt, and enforce a bicycle facilities plan is

found in Acts 1957, No. 186, which is the municipal planning enabling legislation

for the State of Arkansas.

Jurisdiction: The bicycle facilities plan’s effective jurisdiction is within the Greenwood city limits. Upon amendments to the city’s Subdivision Regulations which will

regard bicycle facilities plan implementation techniques, the effective jurisdiction

of the bicycle facilities plan will be the Planning Jurisdiction Area of the city of

Greenwood.

Greenwood Bicycle Plan

Section I: Goals and Objectives:

Goals:

- To incorporate bicycle design and construction considerations in all street reconstruction and new construction projects.

- To provide for the public’s interest, safety, and general welfare by encouraging and offering a well planned, safe, and efficient bikeway system that will interconnect neighborhoods and provide access to both neighborhood and citywide destinations.

- To ensure the safety of the bicyclist and motorist alike by initiating a broad based bicycle educational program in the community.

Objectives:

  - Require all new street construction or reconstruction to accommodate bicycle usage by providing the appropriate bicycle facility type as defined by the Greenwood Bicycle Plan.

  - Develop design and construction specifications for bikeways that are safe and compatible with the city’s streets based on the following criteria:

1. Street function, ie: arterial, collector...

2. Existing and projected vehicular traffic volumes.

3. Actual rather than posted speeds on the city’s streets.

  - Develop a comprehensive bikeway plan for the city that addresses  community needs such as access to schools, parks, commercial centers,  churches; as well as to provide access to employment centers within the community.

  - Locate bicycle facilities within ¼ mile of each neighborhood within the city limits.

  - Decrease traffic growth, and thereby improve automobile traffic flow, through increased use of bicycles and mass transit for transportation.

  - Develop city ordinances for bicycle parking facilities based on the number of car parking spaces available and/or total square foot of the facility.

  - In concert with the Greenwood Police Department, the Greenwood Public School System, the media community, the State Highway and Transportation Department, and local bicycle advocates, initiate a bicycle educational/public awareness program for bicyclists and motorists.

Section II: General Policies Regarding The Bicycle Plan

- Bicycle transportation is an integral part of the overall transportation plan for the city and will be provided the same consideration for implementation as any element of the Greenwood Master Street Plan.

- The Greenwood Bicycle Plan will be implemented through the enforcement of the city’s Master Street Plan, subdivision regulations, and other development codes and ordinances. Where necessary, the city will develop amendments to existing regulations and ordinances to facilitate the orderly implementation of the Bicycle Plan.

- Bicycle transportation improvements are essential elements of the city’s capital improvements programming process and will be afforded the same considerations as other transportation improvements which utilize the city’s sales tax receipts for construction.

- The city of Greenwood considers the Bicycle Plan an important element of the Master Street Plan. Elements of the Master Street Plan are also contained in the Bi-State Transportation Study’s Year 2025 Transportation Plan which is the official regional transportation plan required by the Federal Highway Administration. The city will request that the Bicycle element of the Master Street Plan be incorporated into the Bi-State Year 2025 Transportation Plan and afforded any and all of the same considerations for federal financial assistance as any other proposal in the Bi-State Year 2025 Transportation Plan.

- The city of Greenwood will work with State and Federal transportation and economic development agencies to secure possible grant and other categorical funds appropriate for the Bicycle Plan implementation.

Section III: Bikeway Plan Structure and Format

- Bicycles are allowed on all Greenwood city streets.

- There are five (5) types of bikeway facilities.. They are as follows:

  - Bike Path/Multi-use Path. This is a bikeway that is physically separated from the motorized vehicular traffic along a transportation corridor. Bike paths can either be within the road right-of-way or on their own separate public right-of-way. They are two (2) directional and thus need to be designed accordingly. (Section IV presents the design and construction specifications for each of the bikeway facilities.)

  - Bike Lane. This is a bikeway that is a physical part of the existing road surface. It is striped for exclusive use for bicycles. These bikeways are found on most arterial roads and significant collector streets. Bike Lane facilities are one directional and always in the same direction as the motorized vehicular traffic. Arterial roads or collector streets that contain  bike lane facilities must have them in both directions thus providing two-way directional bicycle travel within the pavement portion of the road or street.  Bike Lanes differ from paved shoulders in that they are in more urban areas and have curbs.

  - Paved Shoulder.  Similar to Bike Lane but in rural areas without curbs.  They are typically not designated with pavement markings although they may be signed.  Cars may be parked here in emergency situations.

  - Wide Outside Lane. This is a bikeway that utilizes an extended curbside vehicular lane width when physical limitation of the existing road surface prohibit a bicycle lane. In this case, automobile parking may be allowed on the same side as the bikeway. The Wide Outside Lane is not striped but may be signed.

  - Shared Roadway. These bikeways comprise the balance of the Greenwood bicycle plan network. In keeping with the policy of the city of Greenwood, any city street or thoroughfare that has not been designated as another type  of bikeway facility or that does not expressly prohibit bicycles such as Interstate Highways and controlled access highways, are designated as Shared Roadway facilities.  These roadways require no striping of the vehicular travel lanes.

- The justification for the predominant use of Shared Roadway facilities is based on the following three (3) points:

  - The physical surface for the bikeway is already in place and will be maintained in accordance with the existing street maintenance program.

  - The existing road and street network is continuous and comprehensive and provides access to the vast majority of destinations in the city.

  - Both the motorists and the bicyclists must adhere to the “rules of the road” as established by Arkansas Statutes and enforced by the Greenwood Police Department. Conformance to these “rules” increases the prospects for safe operations.

Section IV: Design Specifications

The following design and construction standards are in accordance with the Association of State Highway and Transportation Officials (ASHTO). These standards are approved and endorsed by the Federal Highway Administration and the Arkansas State Highway and Transportation Department.

Design Considerations:

- Where a road or street is being constructed or reconstructed, any and all gravel or otherwise unpaved driveway or alley approach should be paved back to a distance of 15 feet to prevent loose gravel or other materials from encroaching into the travel lane/bike lane.

- Where parallel parking is permitted, a bike lane must be located between the parking space and the travel lane and have a minimum width of 5 feet.

- Where a bike lane is planned for a one-way street, it should be located on the right side of the street. In cases where vehicular conflicts or other safety issues may arise, a bike lane may be allowed on the left side only until it is feasible to bring it back to the right side of the street.

- Bikeways may not be placed on existing or planned sidewalks. Pedestrian conflicts should be avoided except where the bicycle facility has been designed for such dual use. Multi-use Path facilities are the only bikeways that may provide for pedestrian and bicycle joint use.

- Two-way bikeways are not allowed except in the case of Bike Path facilities which have, relative to width, been properly designed to accommodate two-way bicycle traffic.

- Angle/diagonal parking should be avoided when future planning bikeway locations. Existing angular parking areas should be studied to identify the impacts of changing this style of parking to parallel parking.

The city of Fort Smith has adopted the standards of the American Association of State Highway and Transportation Officials Bikeway Facilities specifications contained in their report Guide for the Development of Bicycles Facilities published in August 1991 (a newer version is available published in 1999).  Additionally, the standards utilized by the city of Portland, Oregon and the Oregon Department of Transportation are being utilized.  As a part of the Bi-State Metropolitan Planning Organization, the city of Greenwood will follow suit in adoption of the same set of standards.

Section V: Maintenance Program

The Maintenance program portion of the Bicycle Plan will be conducted by the Greenwood Street Department in concert with their regularly scheduled street maintenance activities.  Special attention will be paid to Bike Lanes, which tend to attract roadway debris and gravel.  These may require additional maintenance for safe operation of the bicycle.

Section VI: Educational Program

The educational program of the Bicycle Plan will be developed jointly by the

following:

Greenwood Police Department

Greenwood Parks Committee

Western Arkansas Planning & Development District

Greenwood Public Schools

Local Bicycling Advocates

Information to be disseminated in educational programming will include:

Bicycling related law

Safety standards and practices

Equipment

Riding in traffic

Health benefits

Commuting tips

Basic maintenance

Map of Greenwood Bikeway Facilities

Section VII: Facility Recommendations

Phase 1 Recommendations (2004-2006) (see attached map)

The primary agenda of the first phase is to connect the schools within the city of Greenwood.  This is primarily accomplished through a major corridor connection using Denver Street.  The corridor is supplemented by several short Multi-use trails: Westwood Trail on the west and the Denver Extension and Wells/Easthills Trail on the east.  The School trail leads north from the Denver corridor to bisect the High School and North Main schools.  These provide for a direct connection between all the schools.  A secondary agenda is to establish the state highways as useable routes for intermediate to advanced riders.  A recommendation is made to include fitness elements along the Denver Extension Trail.  Key Destinations along the proposed route in this phase include: all community schools (Westwood, North Main, Greenwood High, Wells, and Easthills), Downtown commercial area, the majority of area churches (including the two largest in town: Greenwood United Methodist Church and First Baptist Church), Sebastian County Fairgrounds, Police Department, Bailey Clinic, Greenwood Performing Arts Center, Coal Miners Historic Monument, South Sebastian Historic Museum, City Hall, and Greenwood Athletic Club.

| Striping and Signage Only |  | Type of Improvement |
| --- | --- | --- |
| 1 | Westwood Avenue | Bike Lane and Signs |
| 2 | Ridgecrest Dr. | Bike Lane and Signs |
| 3 | Whippoorwill Dr. | Wide Outside Lane and Signs |
| 4 | All major crossings/intersections | Establish Crosswalk and Signs |
| New Construction |  |  |
| 1 | Widen Denver St. (Hwy 71 to Bell Road) | Bike Lane and Signs |
| 2 | Westwood Trail | Multi-use Path |
| 3 | Widen 10/10 Spur where necessary (Hwy 71 to eastern city limit) in cooperation with AR-DOT | Paved Shoulder |
| 4 | West Denver Hill Bypass Trail | Multi-use Path |
| 5 | Widen Aster/Bayberry | Bike Lane and Signs |
| 6 | Gary Street Bike Boulevard (Main to Daisy) | Bike Blvd/Pedestrian Mall |
| 7 | Widen Daisy (Denver to Gary) | Bike Lane and Signs |
| 8 | School Trail (El Paso north to Knoxville) | Multi-use Path |
| 9 | Denver Extension Fitness Trail | Multi-use Path |
| 10 | Whippoorwill Extension Trail | Multi-use Path |
| 11 | Wells/Easthills Trail | Multi-use Path |
| Acquisitions |  |  |
| 1 | Land for Trails (Westwood, W. Denver Hill, Denver Extension, Whippoorwill Extension, School Trail access point) | Right of Way, Easements, Purchase, or Donations |

Phase 2 Recommendations (2007-2008)

The key agenda in this phase of development is increasing the interconnectivity of the trails with community neighborhoods and parks.  This is substantially accomplished by adding the Crooked Creek Trail with short neighborhood accesses and the Golf Course Trail.  This includes most neighborhoods throughout the city.  Bell Park, South Sebastian Boys and Girls Club, and the golf course/pool are also accessed through this phase.

| Striping and Signage Only |  | Type of Improvement |
| --- | --- | --- |
| 1 | Sign short stretch of Deerwood Dr. | Sign Only |
| New Construction |  |  |
| 1 | Crooked Creek Trail/neighborhood extensions | Multi-use Path |
| 2 | Widen Webster | Bike Lane and Signs |
| 3 | Widen W. Houston from Jones to Main | Wide Outside Lane and Signs |
| 4 | Widen Main from Bell Park to Denver | Bike Lane and Signs |
| 5 | Widen Atlanta from Coker to Bell Rd. | Bike Lane and Signs |
| 6 | Widen E. Nashville from Main to Division | Wide Outside Lane and Signs |
| 7 | Widen Division from Nashville to Knoxville | Wide Outside Lane and Signs |
| 8 | Widen E. Knoxville from Main to Daisy | Bike Lane and Signs |
| 9 | Widen Daisy from Knoxville to Gary | Bike Lane and Signs |
| 10 | Widen Bass from Denver to Center | Bike Lane and Signs |
| 11 | Twin Bridges Trail | Multi-use Path |
| 12 | Widen Ware Rd | Bike Lane and Signs |
| 13 | Widen Birchwood Ln | Bike Lane and Signs |
| 14 | Country Club Trail | Multi-use Path |
| 15 | Widen Country Club Rd. | Wide Outside Lane and Signs |
| 16 | Widen Old Chismville Rd from 10 Hwy to Deerwood Dr. | Paved Shoulder |
| 17 | Widen Deerwood from Old Chismville to Been Ridge Rd. | Wide Outside Lane and Paved Shoulder |
| 18 | Widen Ulmer from trail N to Meadow Bridge | Paved Shoulder |
| 19 | Widen Meadow Bridge | Paved Shoulder |
| 20 | Widen N Main from Meadow Bridge to Bell Park | Paved Shoulder |
| Acquisitions |  |  |
| 1 | Land for Crooked Creek and Twin Bridges Trails | Right of Way, Easements, Purchase, or Donations |

Phase 3 Recommendations (2009-2010)

A final emphasis is on developing future resources for bike/transit connectivity with Fort Smith.  In addition to the Bike Park & Ride elements, the remaining neighborhoods are added during this phase.  Finally, a recreational fitness trail was included around Greenwood Lake to provide a scenic walking/biking facility for lifelong fitness.

| Striping and Signage Only |  | Type of Improvement |
| --- | --- | --- |
| 1 | Stripe and Sign Walmart entry road | Bike Lane and Sign |
| New Construction |  |  |
| 1 | Park and Ride Spur Tail and Facility (Walmart) | Bike Parking/Transit Access |
| 2 | Widen Bell Park Loop | Paved Shoulder |
| 3 | Widen Mt. Harmony Rd | Paved Shoulder |
| 4 | Widen Hwy 96 south of 10 Hwy to Stewart Ct. | Paved Shoulder |
| 5 | Widen Stewart Ct | Paved Shoulder |
| 6 | Widen Water Tower Rd | Paved Shoulder |
| 7 | Greenwood Lake Trail | Multi-use Recreational Path |
| 8 | Denver/Crooked Creek Bike Park and Ride | Bike Parking/Transit Access |
| 9 | Whippoorwill Bike Park and Ride | Bike Parking/Transit Access |
| Acquisitions |  |  |
| 1 | Acquire Bike Park and Ride Land | Bike Parking |
| 2 | Bike Lockers and Short Term Parking Options | Bike Parking |
| 2 | Shuttle Buses for Bike Park and Ride Users | Mass Transit |